ARM CPU Emulator

Plain-English Guide — What the Code Does & How to Explain It

Software & Systems Development  |  CRN 11550  |  Component 1

# Part 1 — The Big Picture

Imagine a real ARM processor chip sitting on a circuit board. Inside it, billions of transistors flip on and off billions of times per second to execute code. This program pretends to BE that chip — but in software, running on your Mac.

What you built: a program that reads 32-bit numbers (machine code), figures out what each one means, and prints the human-readable equivalent. That process is called disassembly.

💡 Think of it like this Machine code is like a book written in secret numeric codes. Your emulator is the translator that turns codes like E3A00001 into readable sentences like MOV r0,#1.

## The Three Things Every CPU Does — Forever

| Step | What It Means in Your Program |
| --- | --- |
| 1. FETCH | Read the next 32-bit instruction from the memory[] array using the PC (R15) as the address |
| 2. DECODE | Work out what type of instruction it is by examining specific bits (bits 27:26 first, then others) |
| 3. EXECUTE | Carry out the action — in Component 1 you just PRINT it; Component 2 will actually run it |

# Part 2 — C Basics You Need to Know

## Why C?

C lets you work directly with memory, individual bits, and bytes. Python hides all that. For building a CPU emulator you NEED that control — real registers are 32 bits wide and you must manipulate them bit-by-bit.

## Fixed-Width Types

In C you must declare exactly how many bits a variable uses. Your code uses:

typedef uint32_t REGISTER; // exactly 32 bits — matches a real ARM register typedef uint32_t WORD; // exactly 32 bits — one memory word typedef uint8_t BYTE; // exactly 8 bits — the status register

## Arrays — Your Virtual Hardware

REGISTER registers[16]; // 16 slots (R0–R15) — these ARE your CPU's registers WORD memory[1024]; // 1024 words = 4KB of virtual RAM BYTE sr = 0; // 8-bit Status Register registers[0] = 5; // Put 5 into R0 registers[15] += 4; // Advance the Program Counter by 4 (next instruction)

## Your Key Functions at a Glance

| Function | What It Does |
| --- | --- |
| main() | Entry point — sets up memory, loads instructions, runs the fetch-decode loop |
| decodeInstruction() | The dispatcher — reads bits 27:26 and routes to the right decoder |
| decodeALU() | Decodes data processing instructions (MOV, ADD, SUB, AND, CMP, etc.) |
| decodeDT() | Decodes memory access instructions (LDR, STR) |
| decodeBranch() | Decodes branch instructions (B, BL) |
| decodeSWI() | Decodes software interrupt (SWI) |
| decodeOperand2() | Handles Operand 2 — both immediate values and shifted registers |
| parseFile() | Reads hex instructions from a .txt file into memory[] |

# Part 3 — What is Assembly Language?

Assembly is one step above machine code. Each instruction is one action. Your program converts the numeric machine code into these readable text instructions:

| Hex Machine Code | Assembly Mnemonic Your Program Prints | What It Means in English |
| --- | --- | --- |
| E3A00001 | MOV r0, #1 | Put the number 1 into register R0 |
| E3A01002 | MOV r1, #2 | Put the number 2 into register R1 |
| E0802001 | ADD r2, r0, r1 | Add R0 and R1 together, put result in R2 (= 3) |
| E2822005 | ADD r2, r2, #5 | Add 5 to R2, store back in R2 (= 8) |

# Part 4 — Bitwise Operations (The Heart of the Decoder)

Every ARM instruction is one 32-bit number. Different bit positions within that number carry different meanings. Your decoder pulls them apart using bitwise operations.

| Operator | Symbol | What It Does — Plain English |
| --- | --- | --- |
| Bitwise AND | & | Keeps only the bits you want, zeroes everything else. Like a stencil — only bits that are 1 in BOTH values survive. |
| Right Shift | >> | Slides all bits to the right by N places. Used to bring a field to position 0 so you can read its value easily. |
| Bitwise OR | ¦ | Combines two values. Any bit that is 1 in EITHER input becomes 1. Used to SET flags. |
| Bitwise NOT | ~ | Flips every bit. 0 becomes 1, 1 becomes 0. Used with & to CLEAR specific bits. |

## Real Example: Decoding E3A00001 (MOV r0, #1)

In binary, this 32-bit instruction looks like this:

Hex: E 3 A 0 0 0 0 1 Bin: 1110 0011 1010 0000 0000 0000 0000 0001 Bits: [31:28][27:26][25][24:21][20][19:16][15:12][11:0] Cond 00 I OpCode S Rn Rd Operand2 AL 00 1 MOV(13) 0 0000 0000 00000001

Your code extracts each field with this pattern:

int condCode = inst >> 28; // Shift right 28 → get 0xE = 14 = AL (always execute) int bit27 = inst & (1 << 27); // Test bit 27 → 0 int bit26 = inst & (1 << 26); // Test bit 26 → 0 ∴ Data Processing int opcode = (inst >> 21) & 0xF; // Shift right 21, keep 4 bits → 13 = MOV int regDest = (inst >> 12) & 0xF; // Shift right 12, keep 4 bits → 0 = r0 int iBit = inst & (1 << 25); // Test bit 25 → 1 = immediate value int imm = inst & 0xFF; // Keep bottom 8 bits → 1 = the value #1

🔑 The Pattern to Remember Shift Right to bring a field to the bottom. AND with a mask to keep only the bits you want. That's ALL the decoder does — over and over for each field of the instruction.

# Part 5 — How the Program Identifies Instruction Types

Bits 27 and 26 are checked FIRST. They tell you the broad category of instruction:

| Bits 27:26 | Category | Examples |
| --- | --- | --- |
| 0 0 | Data Processing (ALU) | MOV, ADD, SUB, AND, ORR, CMP, TST, EOR, BIC, MVN... |
| 0 1 | Data Transfer (Memory) | LDR (load from RAM), STR (store to RAM) |
| 1 0 + bit25=1 | Branch | B (jump), BL (jump and save return address) |
| 1 1 | Software Interrupt | SWI (call the OS) |

## The 16 ALU Instructions (bits 24:21 = opcode)

| Bits 24:21 | Mnemonic | Operation |
| --- | --- | --- |
| 0000 = 0 | AND | Rd = Rn & Op2 (bitwise AND) |
| 0001 = 1 | EOR | Rd = Rn ^ Op2 (bitwise XOR) |
| 0010 = 2 | SUB | Rd = Rn - Op2 |
| 0011 = 3 | RSB | Rd = Op2 - Rn (reversed subtract) |
| 0100 = 4 | ADD | Rd = Rn + Op2 |
| 0101 = 5 | ADC | Rd = Rn + Op2 + Carry flag |
| 0110 = 6 | SBC | Rd = Rn - Op2 - !Carry |
| 0111 = 7 | RSC | Rd = Op2 - Rn - !Carry |
| 1000 = 8 | TST | Flags only from Rn & Op2 (no result stored) |
| 1001 = 9 | TEQ | Flags only from Rn ^ Op2 (no result stored) |
| 1010 = 10 | CMP | Flags only from Rn - Op2 (no result stored) |
| 1011 = 11 | CMN | Flags only from Rn + Op2 (no result stored) |
| 1100 = 12 | ORR | Rd = Rn ¦ Op2 (bitwise OR) |
| 1101 = 13 | MOV | Rd = Op2 (Rn is ignored) |
| 1110 = 14 | BIC | Rd = Rn & ~Op2 (bit clear) |
| 1111 = 15 | MVN | Rd = ~Op2 (move NOT / bitwise invert) |

## Operand 2 — Two Forms Depending on the I Bit (bit 25)

📌 I = 1 (Immediate) Bits 7:0 = 8-bit immediate value. Bits 11:8 = rotate amount (x2). Examples: #1, #5, #2048. Baked into the instruction itself.

📌 I = 0 (Register) Bits 3:0 = register number. Bits 6:5 = shift type (LSL/LSR/ASR/ROR). Bits 11:7 = how much to shift. Examples: r0, LSR #1 or r4, LSL #2.

## All 16 Condition Codes (bits 31:28)

Every ARM instruction carries a condition code. The CPU checks the Status Register flags before executing. Most instructions use AL (always = 0xE) so they always run.

| Bits 31:28 | Suffix | Execute When... |
| --- | --- | --- |
| 0000 = 0 | EQ | Z=1 (last result was zero / values were equal) |
| 0001 = 1 | NE | Z=0 (last result was not zero / not equal) |
| 0010 = 2 | CS | C=1 (carry was set — unsigned >=) |
| 0011 = 3 | CC | C=0 (no carry — unsigned <) |
| 0100 = 4 | MI | N=1 (result was negative / minus) |
| 0101 = 5 | PL | N=0 (result was positive / plus) |
| 0110 = 6 | VS | V=1 (signed overflow occurred) |
| 0111 = 7 | VC | V=0 (no overflow) |
| 1000 = 8 | HI | C=1 AND Z=0 (unsigned higher) |
| 1001 = 9 | LS | C=0 OR Z=1 (unsigned lower or same) |
| 1010 = 10 | GE | N=V (signed greater or equal) |
| 1011 = 11 | LT | N != V (signed less than) |
| 1100 = 12 | GT | Z=0 AND N=V (signed greater than) |
| 1101 = 13 | LE | Z=1 OR N!=V (signed less or equal) |
| 1110 = 14 | AL | Always — unconditional. Most instructions use this. |
| 1111 = 15 | NV | Never — instruction is never executed. |

# Part 6 — The Status Register (N Z C V)

The 8-bit Status Register (sr) holds four flags in its top 4 bits. These flags are set by comparison and arithmetic instructions, and then read by conditional instructions.

| Bit | Flag | Name | Set When... |
| --- | --- | --- | --- |
| 7 | N | Negative | The result's bit 31 was 1 (the result was negative in signed arithmetic) |
| 6 | Z | Zero | The result was exactly 0 |
| 5 | C | Carry | There was a carry out of bit 31 (unsigned overflow) |
| 4 | V | Overflow | Signed arithmetic produced an impossible result (e.g. adding two positives got a negative) |

The helper functions isSet(STAT_Z), isClear(STAT_N), setFlag(), clearFlag(), and updateFlag() handle all the bit operations on the SR so the rest of the code stays readable.

# Part 7 — LDR, STR, Branch, and SWI

## LDR and STR — Reading and Writing Memory

| Instruction | What It Does | Example |
| --- | --- | --- |
| LDR (Load Register) | Reads a value FROM memory INTO a register | LDR r0, [r1] — load from address in R1 into R0 |
| STR (Store Register) | Writes a value FROM a register INTO memory | STR r4, [r0] — store R4 into address in R0 |

Bit 20 = Load (1) or Store (0). Bit 24 = Pre-index (offset before) or Post-index (offset after). Bit 23 = Add (1) or Subtract (0) the offset. Bit 21 = Write-back (update base register after transfer).

## B and BL — Branching (Jumping)

B: Changes the Program Counter to jump to a new address.  BL: Same, but first saves the current PC+4 into R14 (the Link Register) so you can return. This is how function calls work in ARM.

The 24-bit signed offset in the instruction is in words. Your code multiplies by 4 (shifts left 2) and adds 8 for the ARM pipeline prefetch offset.

## SWI — Software Interrupt

SWI is like asking the operating system to do something for you. The CPU stops, saves the return address to R14 (supervisor mode), and jumps to the SWI handler. The 24-bit code tells the handler what service is needed.

# Part 8 — How the Program Flows

- main() sets all 16 registers to 0. R13 (stack pointer) is set to 0x1000 (top of our 4KB virtual RAM).

- If a filename argument was given (e.g. ./emulator prog1.txt), parseFile() reads the hex codes from that file into memory[].

- Otherwise, the hardcoded test instructions (MOV r0,#1 etc.) are already in memory[].

- The while(!done) loop begins. Each time: memory[R15 / 4] is read to FETCH the instruction.

- R15 is increased by 4 immediately after fetching (pointing to the next instruction).

- decodeInstruction() checks bits 27:26 and calls the correct decoder function.

- The decoder prints the formatted mnemonic. Loop repeats until a 0x00000000 word is found.

# Part 9 — Tutor Questions & Model Answers

"How does your program know it's a MOV instruction?"

Bits 27:26 are both 0, so it's data processing. Bits 24:21 are extracted: 1101 in binary = 13 in decimal. The alu_op_labels array at index 13 is the string "MOV". That's the mnemonic that gets printed.

"What is the Program Counter and how does it advance?"

The PC is R15, stored in registers[15]. Every ARM instruction is exactly 4 bytes (32 bits). After fetching each instruction, the code does registers[15] += 4. We divide by 4 when indexing memory (memory[PC >> 2]) because memory[] is an array of 32-bit words, not bytes.

"What does the I bit do?"

Bit 25 is the Immediate bit. When it is 1, Operand 2 is a constant number baked into the instruction (like #5). When it is 0, Operand 2 comes from a register, optionally shifted. Your decodeOperand2() function handles both cases.

"Why uint32_t instead of int?"

int is platform-dependent — it could be 16, 32, or 64 bits depending on the system. uint32_t from <stdint.h> is ALWAYS exactly 32 bits unsigned, which matches the real ARM register size precisely. This matters when shifting and masking bits.

"What is the S bit?"

Bit 20 is the Set-Status bit. When set to 1, the instruction updates the Status Register flags (N, Z, C, V) based on the result. When 0, flags are unchanged. That's why ADD and ADDS are different — ADDS updates the flags, plain ADD does not. In Assembly the S suffix is printed when this bit is set.

"What would you add for Component 2?"

Instead of just printing the mnemonic, the execute phase would perform the actual operation — for ADD it would add the values and store the result in the destination register. I'd also implement condition code checking: read the SR flags, compare to the condition code, and skip the instruction entirely if the condition isn't met.

# Quick Reference Card

| Term | Plain-English Definition |
| --- | --- |
| Machine code | Raw binary/hex numbers that a CPU reads directly — e.g. E3A00001 |
| Assembly (mnemonic) | Human-readable text version — e.g. MOV r0, #1 |
| Disassembler | Converts machine code back to Assembly. That's what you built for Component 1. |
| Register | An ultra-fast 32-bit storage slot inside the CPU. Your CPU has 16: R0 to R15. |
| Program Counter (R15) | Always holds the address of the next instruction to fetch |
| Stack Pointer (R13) | Points to the top of the stack area in RAM — used for temporary storage |
| Link Register (R14) | Stores the return address when BL (branch with link) is called |
| Status Register (SR) | 8-bit register holding N Z C V flags set by the last arithmetic operation |
| Bitwise AND (&) | Extracts specific bits — zeroes out everything you don't want (masking) |
| Right shift (>>) | Slides bits to the right — used to bring a field to bit position 0 |
| Immediate value | A constant number baked into the instruction itself (not read from a register) |
| Condition code | Top 4 bits of every instruction — determines if it executes based on SR flags |
| Virtual RAM | The memory[] array in your code — simulates real computer memory |
| Operand 2 | The flexible second argument in ALU instructions — immediate or shifted register |
| Sentinel value | The 0x00000000 word at the end of your instruction list — signals end of program |